# Project description

Optimization of a 6-month operating policy (1 June 2026 → 1 December 2026) for a pair of 100-MW data centers in the ERCOT region — Houston (HB_HOUSTON) and West Texas (HB_WEST) — covering allocation of compute capacity between training and inference, timing of new model releases, and procurement of grid power. The framework combines a deterministic linear program (LP) solving each hour's decisions for a given (training cadence, procurement) policy with a Monte-Carlo wrapper that simulates joint price paths and selects the policy maximizing expected profit under uncertainty.

# Project setup

## 3 main decisions

- Training vs. inference compute split each hour at each site

- Power procurement at each site: RT LMP vs. tolling (Houston only — no gas plant available at West) and BESS dispatch (charge from grid, discharge to data center, or sell back to grid)

- Frequency of frontier-model retraining (cadence in days). Higher cadence keeps the deployed model closer to peak revenue but takes compute away from inference; lower cadence preserves inference capacity but lets revenue decay

# Assigned Tasks and Deliverables

The prescribed task entails employing real-options analysis to design an economically viable operating policy for allocation and timing for incurring training cost as well as power procurement during the next 6 months (starting 1 June 2026 to 1 December 2026) with corresponding profitability (i.e., volatility-dependent conditional payoff) estimates during this period.

- Detailed description of the process of analysis, motivation/rationale justifying the methodology, and all assumptions.

- Value and estimate breakeven as well as marginal profitability increases for tolling fixed-lease-rate battery storage (BESS) at each location with recommendations as to under what market conditions either or both of these tolling arrangements would be economically viable.

- Substantive description of wind-solar intermittency in the context of MIHR citing the ERCOT data portal, and the contingency of the prescribed unscheduled outage scenarios.

# Training + Inference

## Training

### Power and computational requirements

Each model release k has a compute requirement that we project from its training-start date in three steps: training_start_date → projected parameter count → projected training FLOPS → required compute-MWh.

### Parameter projection

Parameter counts in notable AI systems have grown exponentially. We fit a log-linear regression of log10(params) on Excel-serial date using the Epoch AI "Parameters in notable artificial intelligence systems" dataset with a post-2020 filter. The post-2020 cut produces a steeper, more recent trend that better matches frontier model sizes; the post-2010 alternative is documented in the code (`fit_growth_curves.py`) for comparison.

Post-2020 fit (n = 395 observations, R² ≈ 0.30):

log10(params) = −49.6585 + 1.32043 × 10⁻³ · day_serial

That gives a parameter-count doubling time of ~228 days. We then apply a 5× competitiveness multiplier (planning-doc convention) so projections track 2026 frontier-class models (DeepSeek 1.6T, GPT-class 1.7T, etc.) rather than the dataset-wide average — the regression line captures the average researcher, including small academic releases that drag the trend down, while we have to compete at frontier scale.

Projected post-multiplier parameter counts at planning-doc reference dates:

| Release | Training start | Params (post-5×) | Pre-multiplier |
| --- | --- | --- | --- |
| R1 (initial) | 2026-06-01 | 1.02 × 10¹² | 2.04 × 10¹¹ |
| R2 | 2026-06-22 | 1.09 × 10¹² | 2.18 × 10¹¹ |
| R3 | 2026-08-22 | 1.31 × 10¹² | 2.62 × 10¹¹ |
| R4 | 2026-10-22 | 1.58 × 10¹² | 3.16 × 10¹¹ |
| R5 | 2026-12-22 | 1.90 × 10¹² | 3.80 × 10¹¹ |

These figures are derived in `assumptions.project_params(training_start_date)`.

### FLOPS projection

Training compute follows a power law in parameter count. We fit log10(petaFLOPS) vs. log10(params) on the Epoch AI "Training computation vs. parameters" dataset (n = 457, R² ≈ 0.84):

log10(petaFLOPS) = −9.8192 + 1.7187 · log10(params)

A 1.72 exponent matches Chinchilla-style scaling (FLOPS ∝ params × tokens, with tokens ∝ params). With the post-2020 parameter projection and the 5× multiplier, the projected training-FLOPS per release is:

| Release | Training start | Training petaFLOPS | FLOPS |
| --- | --- | --- | --- |
| R1 | 2026-06-01 | 6.64 × 10¹⁰ | 6.64 × 10²⁵ |
| R2 | 2026-06-22 | 7.41 × 10¹⁰ | 7.41 × 10²⁵ |
| R3 | 2026-08-22 | 1.02 × 10¹¹ | 1.02 × 10²⁶ |
| R4 | 2026-10-22 | 1.40 × 10¹¹ | 1.40 × 10²⁶ |
| R5 | 2026-12-22 | 1.93 × 10¹¹ | 1.93 × 10²⁶ |

Derived in `assumptions.project_petaflops(training_start_date)`.

### Hardware and FLOPS → MWh conversion

Each data center holds 90,000 H-100-class GPUs at 80 MW of compute capacity (180,000 GPUs and 160 MW compute across both sites). H-100 SXM5 and H-100 PCIe have different sustained throughputs; SXM is generally better for training. We model an aggregate effective rate built from a configurable SXM/PCIe split:

| Parameter | Default value | Notes |
| --- | --- | --- |
| H100_SXM_SUSTAINED_TFLOPS_PER_SEC | 500 TF/s | ≈ 50 % of FP8-dense theoretical 989 TF/s. ⚠ TBD |
| H100_PCIE_SUSTAINED_TFLOPS_PER_SEC | 380 TF/s | ≈ 50 % of FP8-dense theoretical 756 TF/s. ⚠ TBD |
| SXM_FRACTION_DEFAULT | 0.60 | 60 % SXM / 40 % PCIe. ⚠ TBD |
| FLOPS_PER_COMPUTE_MWH | 1.83 × 10²¹ | Derived: GPUs × (split · TF/s) × 3600 / 80 MW |

Aggregate sustained throughput at 100 % compute utilization across both sites = 81.4 PFLOP/s = 2.93 × 10²³ FLOPS/hour. Dividing the per-release FLOPS by FLOPS_PER_COMPUTE_MWH gives the compute-MWh needed for that release. Multiplying by PUE = 1.25 converts to grid-MWh, which is what the LP's `train` and `inf` variables track.

### Three core decisions, recap

- Training cadence (days between releases). Treated as a discrete sweep over candidate values, evaluated in expectation over MC price paths.

- SXM vs PCIe split. Currently collapsed into a single aggregate FLOPS_PER_COMPUTE_MWH because the underlying per-workload sustained rates are flagged TBD in the planning doc. Easy to split when finalized.

- Hourly training/inference split per site. The LP picks these endogenously given the cadence and price paths.

### First release: 100 % compute

The initial release R1 begins 1 June 2026 and runs at 100 % compute utilization on both sites — no inference revenue accrues during R1's window. R1's window length is therefore set by compute-need / max-compute-rate:

R1_duration = project_compute_mwh(2026-06-01) / (2 × 80 MW · 24 h) ≈ 36,289 / 3,840 ≈ 9.5 → 10 days

All subsequent releases R(k+1) start at R(k)'s release date and run for the chosen cadence_days, with both training and inference allowed in those windows.

## Inference

### Basic constraints

- Inference can only use compute capacity not currently being used for training. Implemented as the LP's per-site compute cap: train + inf ≤ 100 grid-MWh/hr (= 80 compute-MWh/hr).

- Token revenue at any hour must cover marginal power cost. The LP enforces this implicitly via its profit objective: an additional MWh of inference is only taken if its revenue exceeds its grid cost.

- Maximum request rate is such that power/computational cost does not exceed that unused by training, in any hour.

### Revenue per MWh

Per the RFP, a 100 % allocation of 80 MW compute to inference produces 5 trillion tokens/day per site. This pins our conversion factor:

TOKENS_PER_COMPUTE_MWH = 5 × 10¹² / (80 × 24) ≈ 2.604 × 10⁹ tokens/cMWh

Token pricing follows GPT-5.4Pro on benchlm.ai with the RFP-specified 2/3 input + 1/3 output revenue mix:

blended rate = (2/3) × $30 + (1/3) × $180 = $80 per million tokens

That implies an inference revenue per compute-MWh of $208,333 (at peak, no decay); after PUE conversion, $166,667 per grid-MWh. This is the peak revenue an hour earns when the deployed model is at its newest. Per-hour revenue then declines via the token-decay path described below.

### Token-price decay between releases

From the planning-doc observation that frontier token prices halve every couple of months, we model the per-hour inference revenue rate as:

rev_inf[h] = base_rev x multiplier_of_current_release x 0.5 ^ (days_since_release / TOKEN_PRICE_HALFLIFE_DAYS)

Where `current_release` is the most-recent release as of hour h. Halflife defaults to 270 days, anchored to benchlm.ai's LLM Pricing Trends Price Index: the curated frontier index fell from 100 (GPT-4 launch March 2023) to 5.5 (April 2026), a 94.5 percent decline over ~37 months, equivalent to log2(100/5.5) ~= 4.18 halvings, so halflife ~= 37/4.18 ~= 8.85 months ~= 270 days. See https://benchlm.ai/llm-pricing-trends. The planning doc's prior 60-day heuristic was ~4.5x faster than this measured rate; it is retained as the fast-decay extreme. Sensitivity is bracketed over {60, 120, 180, 270, 360, 540} days x cadence in `halflife_sensitivity.py`.

### Per-release multipliers (token cost / quality)

Each release carries a `token_revenue_multiplier`. Four schemes are available, configurable per-run:

| Scheme | Multiplier rule | Captures |
| --- | --- | --- |
| constant | 1.0 for every release | No generation-over-generation differentiation; only the decay sawtooth matters |
| quality_uplift | uplift_factor^k for release k | Each new release captures more value because it's a better model |
| market_decay | 0.5^(release_t_days/270) | Each new release earns less because the consumer market price has fallen |
| doc_blended | quality_uplift × market_decay | Planning-doc story: quality rises but market deflates simultaneously |

Default for the headline run is `doc_blended`. The uplift_factor is now CADENCE-DEPENDENT: uplift_factor = 2 ** (period_days / 210), i.e. metr_uplift_factor(period_days) in assumptions.py. This stays METR-consistent across the Phase A cadence sweep: METR measured that frontier-model task-length capability doubles every ~7 months (210 days), and a faster cadence ships less capability gain PER release (but more releases). Concrete values: 30d -> 1.105x; 60d -> 1.219x (the planning-doc-bimonthly anchor point); 90d -> 1.346x. The earlier fixed default UPLIFT_FACTOR_DEFAULT = 1.22 was implicitly anchored to 60d cadence and overstated the per-release growth at the actual 30d optimum (the cadence winner shifted from 30d to ~95d when this was corrected). See https://metr.org/blog/2025-03-19-measuring-ai-ability-to-complete-long-tasks/.

### Revenue at the hour level

Total inference revenue per hour at a site, given the LP picked inf grid-MWh of inference:

revenue[h, s] = inf[h, s] × rev_inf[h]

Aggregated across hours and sites and netted against power costs (LMP/toll/BESS-charge) and BESS lease (if applicable), this is the LP's objective.

# Conversion (compute → grid-MWh)

The LP's training and inference decision variables are in grid-MWh — the same units as the power-procurement variables — because compute capacity converts to grid power at PUE = 1.25. Key relationships:

| Quantity | Formula | Value (default) |
| --- | --- | --- |
| Power Use Effectiveness | Grid MWh / Compute MWh | 1.25 |
| Per-site grid power cap | RFP | 100 MWh/hr |
| Per-site compute capacity | Grid cap ÷ PUE | 80 compute-MWh/hr |
| Both-site grid cap | 200 MWh/hr | = 4,800 MWh/day |
| Both-site compute cap | 160 compute-MWh/hr | = 3,840 cMWh/day |
| FLOPS per compute-MWh | Σ_GPU type frac · TF/s · 3600 / 80 MW | 1.83 × 10²¹ |
| Tokens per compute-MWh | 5T tokens/day ÷ (80 MW · 24 h) | 2.60 × 10⁹ |

Constraints (planning doc):

- Per-site grid draw g_lmp + g_toll ≤ 100 MWh/hr × capacity_factor

- Aggregate grid draw across both sites ≤ 200 MWh/hr

- Per-site train + inf ≤ 100 grid-MWh/hr (≡ 80 compute-MWh/hr after PUE)

- Aggregate train + inf ≤ 200 grid-MWh/hr (≡ 160 compute-MWh)

- Compute-clearing: g_lmp + g_toll + dis_dc = train + inf (LP balance equation)

- RFP daily training floor: Σ train per day, summed across both sites, ≥ 500 MWh-grid

# Power

## Grid LMP (HB_HOUSTON, HB_WEST)

Hourly Day-Ahead Market settlement-point prices for the two ERCOT hubs are sourced from the ERCOT ISO publishing site (xlsx in `data/`). For the 6-month horizon, we use the actual 2025 June-November DAM hourly prices, shifted forward one calendar year, as a deterministic proxy. For the headline real-options analysis these prices are replaced by N Monte-Carlo paths simulated from a seasonal Ornstein-Uhlenbeck model calibrated on the full 2025 actuals.

Note: the RFP requests RT-LMP. DAM is the closest available proxy in the repo; RT is typically more volatile and would slightly increase the value of tolling and BESS arbitrage.

OU sigma calibration: the per-series sigma is fit with method `tail_q` at `tail_quantile=0.01` (anchors the OU stationary Gaussian to the empirical residual at the 1st-percentile). This fixes the seasonal-OU underdispersion at HB_WEST (the seasonal table absorbs wind-driven negative-price hours into seasonal means, leaving an underdispersed residual). Under the default, simulated HB_WEST negative-hour frequency rises from 0.90 % (legacy `mle` fit, identical to the ltemry/FTG-Final-Project port) to 3.07 %, within ~20 % of the 2025 actual 3.85 %. Both methods are available via the --calibration {mle|tail_q} CLI flag; mle is retained for direct parity with ltemry's analysis when needed.

## Henry Hub gas (for tolling)

Daily Henry Hub spot prices from the EIA are used in the Houston tolling cost formula. Loaded from `HH_full.csv`.

## Tolling at Houston (period-long option, hourly exercise)

Tolling is structured as a binary period-long contract decision (`Scenario.use_houston_tolling`). When the contract is signed, the LP gets an hourly option to draw up to TOLL_MAX_MW of power at the tolling cost — exercised only when LMP > toll cost. Toll variable cost per MWh:

toll_cost[$/MWh] = (HH_gas[$/MMBtu] + $3/MMBtu O&M) × 9.5 MMBtu/MWh ≈ $57/MWh at $3 gas

Heat rate is 9,500 BTU/kWh per the RFP — typical of an efficient simple-cycle peaker. West Texas has no tolling option (no gas plant available).

Tolling parameters. The capacity payment scales linearly with the buyer's reserved MW (a flat $ cost over the 6-month horizon, applied at the procurement-comparison level alongside the BESS lease). Two sweeps quantify the seller-vs-buyer gap: --capacity-payment-sweep varies the $/kW-mo rate at fixed 100 MW; --reservation-sweep solves the LP at MW ∈ {0, 20, 40, 60, 80, 100} so the buyer's optimal ex-ante reservation can be picked given the rate. Cross-check against ltemry/FTG-Final-Project independently places gross toll value at ~$1.4M / 6mo, well below the default $4.8M at 100 MW × $8/kW-mo.

| Parameter | Default | Notes |
| --- | --- | --- |
| TOLL_HEAT_RATE_BTU_PER_KWH | 9,500 | RFP — simple-cycle peaker |
| TOLL_VOM_PER_MMBTU | $3/MMBtu | RFP — gas premium above HH spot (applied inside the toll's variable cost) |
| TOLL_MAX_MW | 100 MWh/hr | Hourly cap = site grid cap (assumption) |
| Scenario.toll_max_mwh_per_day | None (unconstrained) | Daily MWh cap on Houston toll. Empirical brackets in assumptions.py: TOLL_DAILY_CAP_PEAKER=720, _INTERMEDIATE=1500, _NEAR_NAMEPLATE=2280 (anchored to EIA SCGT capacity-factor history). --toll-cap-sweep runs all four on the sweep driver. |
| Scenario.toll_mw_reserved | None ⇒ 100 MW | Buyer-side MW reservation, committed ex ante (before paths realize). The LP caps g_toll at this MW × capacity_factor and the capacity payment scales linearly. Outer sweep via --reservation-sweep on power_procurement_sweep.py. |
| TOLL_CAPACITY_PAYMENT_PER_KW_MONTH | $8/kW-mo | SELLER-SIDE market rate (RFP didn't specify). $4.8M for the 6-month horizon at 100 MW. Midpoint of $5–$15/kW-mo SCGT range from ERCOT public IPP disclosures (Calpine/Vistra/NRG 10-Ks). Buyer-side gross toll value ≈ $1.14–1.21M / 6mo across our 4 drift scenarios (independently confirmed against ltemry/FTG-Final-Project at $1.42M), so the default seller rate doesn't clear by ~4×. |

## BESS (40 MW / 160 MWh / 92 % RTE at each site)

Battery storage is modeled as a tolling agreement (physical or virtual) for 6 months. The contract is binary per site (`Scenario.bess_sites` can be any subset of {HOUSTON, WEST}). When active at a site, the LP gets four hourly decisions:

- ch[h, s]: MWh charged from the grid (drawn at LMP)

- dis_dc[h, s]: MWh discharged to the data center (substitutes for grid draw at LMP)

- dis_grid[h, s]: MWh discharged and sold back to the grid at LMP

- soc[h, s]: state of charge at hour h, bounded 0 ≤ soc ≤ 160 MWh

BESS dynamics:

soc[h+1] = soc[h] + √0.92 × ch[h] − (dis_dc[h] + dis_grid[h]) / √0.92

Charge and combined discharge each capped at 40 MWh/hr; round-trip efficiency 92 % (= per-direction √0.92 ≈ 0.959). The 6-month lease per site is amortized from the RFP-stated cost structure:

lease/site = $60M capex / 15 yr / 2 + $2M opex/yr / 2 = $3M for 6 months

Or $6M total if BESS is active at both sites. This fixed cost is excluded from the LP objective (it's a constant — constants don't change dispatch decisions) and applied at the procurement-comparison level in compute_breakdown so per-scenario profit numbers reflect it. The Houston toll capacity payment ($4.8M for the 6-month horizon at the $8/kW-mo default) is treated symmetrically.

# Model Dynamic

## Decision variables

Per hour h ∈ {0..4391} and per site s ∈ {HOUSTON, WEST}:

| Variable | Units | Bounds | Meaning |
| --- | --- | --- | --- |
| g_lmp[h, s] | grid-MWh | 0 .. 100 | Power drawn at RT LMP |
| g_toll[h, s] | grid-MWh | 0 .. 100, 0 at WEST | Power drawn under Houston tolling |
| train[h, s] | grid-MWh | ≥ 0 | Compute allocated to training |
| inf[h, s] | grid-MWh | ≥ 0 | Compute allocated to inference |
| ch[h, s] | MWh | 0 .. 40 (BESS sites) | BESS charge from grid |
| dis_dc[h, s] | MWh | ≥ 0 | BESS discharge to data center |
| dis_grid[h, s] | MWh | ≥ 0 | BESS discharge sold at LMP |
| soc[h, s] | MWh | 0 .. 160 | BESS state of charge |

## Model-variable parameters

- Training cadence (days between releases). Default candidate set: [10, 15, 20, 25, 30, 45, 60, 75, 90, 120, 150, 180]. Filter retains those whose smallest non-initial release's natural training rate is ≥ 500 MWh-grid/day AND whose largest release's required rate is ≤ 4,800 MWh-grid/day. Typical filtered set: [25, 30, 45, 60, 75, 90].

- Token-multiplier scheme (per-release revenue scaling). Default `doc_blended`.

- Procurement scenario (use_houston_tolling, bess_sites). Default everything-on; Phase C optimizes.

- Number of Monte-Carlo paths. Default 50 for production runs.

## Objective

Maximize expected profit across MC price paths:

max  Σ_h Σ_s ( rev_inf[h] · inf[h, s]  +  LMP[h, s] · dis_grid[h, s] )

− Σ_h Σ_s ( LMP[h, s] · g_lmp[h, s]  +  toll_cost[h] · g_toll[h, s]  +  LMP[h, s] · ch[h, s] )

Fixed costs (BESS_6mo_lease × |bess_sites|, TOLL_6mo_capacity_payment × use_houston_tolling) are NOT in the LP objective above — they're constants from the LP's perspective and don't affect hourly dispatch. They're netted into profit at the procurement-comparison level in compute_breakdown.

## Constraints

- Per-site grid draw cap: g_lmp + g_toll ≤ 100 MWh/hr × capacity_factor

- Per-site compute cap: train + inf ≤ 100 MWh-grid/hr (= 80 compute-MWh/hr)

- Power balance: g_lmp + g_toll + dis_dc = train + inf (LP balance; ensures power into the DC matches compute load)

- Tolling at Houston only: g_toll[h, WEST] = 0

- Houston toll capacity: g_toll[h, HOUSTON] ≤ TOLL_MAX_MW (× capacity_factor)

- Optional daily toll MWh cap: Σ_h g_toll[h, HOUSTON] per day ≤ Scenario.toll_max_mwh_per_day (peaker 720 / intermediate 1500 / near-nameplate 2280 — EIA-anchored brackets)

- BESS power: ch ≤ 40 MWh/hr, dis_dc + dis_grid ≤ 40 MWh/hr

- BESS SOC: 0 ≤ soc ≤ 160 MWh, soc[h+1] = soc[h] + √0.92 × ch − (dis_dc + dis_grid) / √0.92

- BESS starts and ends at SOC = 0 (no inventory at horizon endpoints)

- Per-release training: Σ_h Σ_s train[h, s] over R_k's window ≥ project_compute_mwh(R_k.start) × PUE

- R1 initial: inf[h, s] = 0 for all h in R1's window (100 % compute → training)

- Outside any release window: train[h, s] = 0 (unless schedule is empty, in which case the daily floor governs)

- RFP daily training floor (mandatory): Σ_s,h-in-day train[h, s] ≥ 500 MWh-grid/day

## Optimization workflow (run_planning_doc.py)

Profit is maximized in four sequential phases:

### Phase A — Cadence selection (procurement = all on)

For each candidate cadence c (after filter), solve the LP across N MC price paths in parallel via a ProcessPool. Mean per-path profit is the score for c. Stage 1 sweeps the filtered broad set; Stage 2 refines six cadences ±30 % around the Stage-1 winner. Winner is the cadence with highest mean profit.

### Phase B — Locked-cadence diagnostic (procurement = all on)

At the Phase-A winning cadence, re-solve the LP on every path and report averaged-across-paths metrics: cost breakdown, profit distribution, procurement mix. This is a marginal-cost view — the LP "uses" BESS even when the lease is net-negative because the lease is a sunk constant from the LP's perspective.

### Phase C — Procurement optimization (full-cost view, lease included)

At the locked cadence, sweep eight procurement combinations: toll ∈ {off, on} × BESS-sites ∈ {none, Houston, West, both}. For each combo, solve N paths and average profit including the BESS lease as a fixed-cost subtraction. Pick the combo with highest mean profit; this is the optimal procurement under full-cost accounting.

### Verification — Cadence under optimal procurement

Re-solve a tight ±30 % cadence neighborhood under Phase C's optimal procurement. Confirms the cadence winner doesn't shift when negative-NPV options are stripped out (in practice never does, because cadence-vs-cadence profit gaps are ~$3 B while procurement-vs-procurement gaps are ~$5 M).

### Final output

Re-solve the LP on every MC path with the (verified cadence, optimal procurement) policy and save:

- Per-path hourly schedule in long format (all decision variables: g_lmp, g_toll, train, inf, compute-MWh, FLOPS, tokens, BESS dispatch if enabled).

- Averaged-across-paths hourly schedule with ±std for every variable.

- Per-cadence mean/std/percentile summary CSV.

- Four result charts auto-rendered into model/outputs/figures/ via plots.py: train/inf diurnal profile, attributed power cost per day for train vs inf, daily procurement mix (and BESS dispatch when active), and LMP/toll-cost overlay with daily toll-exercise frequency. Regenerated on every headline run; failures are caught and logged so the LP itself is unaffected.

- Console: first 24 hours at HOUSTON with mean ± std annotations.

## Inputs at a glance

| Group | Item | Source / Default |
| --- | --- | --- |
| Power | Hourly RT LMP (HB_HOUSTON, HB_WEST) | ERCOT DAM 2025, shifted +1 yr; MC paths from OU calibration |
| Power | Tolling cost | (HH_daily + $3/MMBtu) × 9.5 MMBtu/MWh |
| Power | BESS 6-month lease per site | $3 M (= $60 M / 15 yr / 2 + $2 M / 2) |
| Power | Facility cap | 200 MWh/hr (= 100/site) |
| Compute | Compute cap | 160 cMWh/hr (= 80/site after PUE 1.25) |
| Compute | Hardware throughput | FLOPS_PER_COMPUTE_MWH = 1.83 × 10²¹ |
| Compute | Min daily training | 500 MWh-grid/day, system-total (mandatory) |
| Training | Param projection | log10(P) = −49.66 + 1.32e-3·day (post-2020 fit, 5× multiplier) |
| Training | FLOPS projection | log10(petaFLOPS) = −9.82 + 1.72·log10(P) |
| Training | R1 fixed | Starts 6/1/2026, 100 % compute, ~10 days |
| Revenue | Token price (peak) | $30/MM input + $180/MM output ⇒ $80/MM blended |
| Revenue | Tokens per compute-MWh | 2.604 × 10⁹ |
| Revenue | Decay halflife | 60 days (sensitivity available) |

# Outstanding Work: Parameters and Assumptions to Finalize

Items below are open questions whose resolution will sharpen the model. Each is wired into the code with a sensible default so the current results are usable, but the defaults should be revisited once authoritative numbers are available.

## Hardware throughput (FLOPS → MWh conversion)

- H-100 SXM sustained throughput — currently 500 TF/s (≈ 50 % of FP8-dense theoretical 989 TF/s). Need an authoritative sustained-FP8 number for our intended training workload (mix of attention, MLP, comms overhead).

- H-100 PCIe sustained throughput — currently 380 TF/s (≈ 50 % of FP8-dense 756 TF/s). Same caveat as SXM.

- SXM vs PCIe split — currently 60 / 40. Drives the aggregate FLOPS_PER_COMPUTE_MWH; today it is collapsed into a single number. Once the split is set, we can also split the LP's `train` variable per hardware class (SXM preferred for training, PCIe biased to inference).

## Inference revenue model

- Token-price projection — currently $30/MM input + $180/MM output blended via the RFP-stated 2/3 + 1/3 mix to $80/MM. This is the spot benchlm.ai rate for GPT-5.4Pro; need a forward-looking curve, not a single number, to better match the 6-month horizon.

- Tokens per request — not currently in the model. The RFP requests it; we use a per-MWh tokens conversion instead. Adding it would let us express revenue at the request level (price × requests/hr) rather than the MWh level.

## Tolling contract

- Scenario.toll_max_mwh_per_day — daily MWh cap on Houston tolling. Three empirically-anchored brackets exposed in assumptions.py: peaker (720 MWh/day, 30 % of nameplate × 24 h — matches EIA's 9.6–14.1 % SCGT capacity-factor range), intermediate (1,500 MWh/day, ~63 % — IPP load-following toll convention, recommended headline default), and near-nameplate (2,280 MWh/day, ~95 % — availability-only haircut). --toll-cap-sweep on power_procurement_sweep.py reports marginal toll value as a function of the cap.

- Capacity payment + reservation sizing — TOLL_CAPACITY_PAYMENT_PER_KW_MONTH defaults to $8/kW-mo (SELLER-SIDE market rate, midpoint of $5–$15/kW-mo SCGT range from ERCOT public IPP disclosures). RFP doesn't specify a value. The buyer's MW reservation is now a model parameter (Scenario.toll_mw_reserved): the LP caps g_toll at the reserved MW and the capacity payment scales linearly. Two sweeps in power_procurement_sweep.py: --capacity-payment-sweep varies K at fixed 100 MW (finds K* ≈ $2/kW-mo across all 4 drift scenarios); --reservation-sweep solves the LP at MW ∈ {0, 20, 40, 60, 80, 100} so the buyer's ex-ante optimal reservation can be picked at any K. Cross-check against ltemry/FTG-Final-Project independently confirms gross toll value ≈ $1.4M / 6mo, well below the default $4.8M seller rate at 100 MW.

- Heat rate — currently 9,500 BTU/kWh per the RFP. This is consistent with an efficient simple-cycle peaker; assumed firm.

## BESS contract

- Lease structure — currently $60 M capex amortized over 15 yr + $2 M opex/yr, half-year prorate ⇒ $3 M / site / 6 months. Need confirmation that this is the actual tolling-style lease structure for the prescribed BESS rather than an outright-buy amortization.

- Round-trip efficiency — currently 92 % (√ on each leg ≈ 0.959). RFP-stated; assumed firm.

- Degradation — not currently modelled (6-month horizon is short enough that this is plausibly second-order, but should be noted).

## Power-price input

- DAM vs RT LMP — RFP requests RT-LMP; we use DAM as the closest available proxy (the project repo's CSVs are DAM). RT is typically more volatile, which would slightly increase the value of both tolling (option-like) and BESS arbitrage.

- Forward-curve drift — MC paths are simulated from OU calibrated on 2025 actuals. Drift is now a CLI knob (--gas-drift-pct, --power-drift-pct on run_monte_carlo.py / run_planning_doc.py / power_procurement_sweep.py) that adds a log-space shift to each series' long-run mean via monte_carlo.apply_drift(). Baseline 2026 = 0 % both, consistent with EIA May-2026 STEO (HH 2026 forecast $3.50/MMBtu vs 2025 actual $3.53). Geopolitical oil overlay: a +30 % Brent shock translates to gas_drift_pct ≈ 0.06 (Brent→HH elasticity ≈ 0.2 via LNG-pull) and power_drift_pct ≈ 0.03 (HH→LMP elasticity ≈ 0.5 via gas-on-margin pass-through).

- Number of MC paths — currently default 50 for production runs. 200+ would tighten confidence intervals on procurement decisions where the gap is small (Phase-C gaps are ~$5 M while paths-stdev is ~$30 M).

## Constraints from the RFP — partially covered

The FTG repo (ltemry/FTG-Final-Project) contains `src/phase4_intermittency_stress.py`, which addresses intermittency and tail-risk events indirectly — by reading them out of LMP signatures rather than modelling wind/solar generation directly. We have vendored the OU calibration, MC simulator, AND (as of this revision) the stress-overlay logic from that repo. What's covered vs what's still open:

- Unscheduled-outage / Uri-style stress (ported, optional toggle) — `model/stress.py` exposes four scenarios from FTG phase 4: none (default), mild (72 h, $200–400/MWh, p = 0.50), moderate (96 h, $500–1500/MWh + $20/MMBtu gas, p = 0.20), uri_full (100 h, $5000–9000/MWh + $250/MMBtu gas, p = 0.05). When `run_planning_doc.py --stress <name>` is set, the chosen overlay is applied to a fraction of the MC paths AFTER the OU simulation and BEFORE the LP solve, on both ERCOT hubs simultaneously (system-wide scarcity event). Default is `none`, so headline results are unchanged. Use it as a sensitivity / stress test — expectation is that the toll option's value rises materially under moderate/uri_full.

- Wind-solar intermittency / MIHR (qualitative — still to report) — phase 4 in the FTG repo computes diurnal LMP percentile profiles, the evening-ramp premium (hour-18 − hour-14, capturing solar dropoff), and negative-LMP frequency at HB_WEST (wind-oversupply proxy). These are LMP-implied signatures of renewable variability, which is consistent with the RFP's request for a substantive discussion citing ERCOT data. Action item: include these diagnostics in the report; the analysis logic exists, it just isn't a separate module in our codebase yet.

- Capacity-factor derate (true MIHR-style — still open) — distinct from the LMP-signature view above. Our LP currently assumes capacity_factor = 1.0 (full 100 MW/site at every hour). A direct MIHR model would multiply the per-site grid cap by a stochastic capacity_factor[h] ≤ 1 driven by renewables + transmission. Not implemented; phase 4 doesn't do this either (it works on prices, not capacity).

# Data sources and citations

Vendored data files (used directly by the calibration / LP):

- data/HH_full.csv - Henry Hub daily spot price ($/MMBtu), EIA Natural Gas Weekly. Source: eia.gov/dnav/ng/hist/rngwhhdD.htm.

- data/rpt.00013060.0000000000000000.DAMLZHBSPP_2025.xlsx - ERCOT 2025 Day-Ahead Market hourly settlement-point prices, HB_HOUSTON and HB_WEST. Source: ercot.com (Market Information / DAM SPP report).

- data/artificial-intelligence-parameter-count.csv - Epoch AI parameter-count time series. Source: epochai.org (Notable AI Models dataset), distributed by Our World in Data under CC BY.

- data/ai-training-computation-vs-parameters-by-researcher-affiliation.csv - Epoch AI training-compute vs parameters panel. Source: epochai.org, distributed by Our World in Data under CC BY.

- data/benchlm-pricing.csv - benchlm.ai snapshot of current API pricing for 100 LLMs across 12 creators (model, creator, input/output $/M tokens, context window, source-type). Source: benchlm.ai/llm-pricing. Used as the empirical price level for the project's revenue model and to fit the per-tier decay slope for HALFLIFE.

- data/model-release-dates.csv - curated release dates for 44 of the 100 benchlm models (Anthropic, OpenAI, Google, DeepSeek, xAI, Z.AI tiers covered through April 2026). Combines a research agent's training-data lookup with WebFetch verification of recent Anthropic announcements. Schema: model, creator, release_date, is_estimated, source. 56 models remain undated, mostly late-2026 placeholders.

- data/benchlm-pricing-with-dates.csv - convenience merge of the two CSVs above, joined on the `model` column. This is what the README's decay-rate fits actually consume.

- data/revenue_model_inputs/ - independent revenue-model anchor dataset added 2026-05-23. Contains frontier_llm_price_observations.csv (10 frontier launches GPT-4 8K through GPT-5.4 Pro, blended 80/20 in/out USD/M), revenue_model_assumptions.csv (recommended values + sensitivities + LogNormal distributions for halflife and uplift), revenue_pricing_scenarios.csv (5 pricing tiers from standard $80/M to priority $160/M), and the source README. Used as a corroborating source for the halflife and uplift anchors derived independently from benchlm.ai's Price Index and METR's task-length doubling.

Cited inline in the planning doc (Epoch AI / OWID series and token-price model):

- Epoch AI - Notable AI Models dataset. Primary source for both vendored CSVs above; the post-2020 log-linear param fit and the params -> petaFLOPS power-law fit are derived from this panel via fit_growth_curves.py. epochai.org/data/notable-ai-models

- Our World in Data - Parameters in notable artificial intelligence systems. The chart format the planning doc reproduces; proximate attribution for the Epoch AI CSV used in our fits. ourworldindata.org/.../artificial-intelligence-parameter-count

- benchlm.ai - LLM pricing index. Source for the GPT-5.4Pro frontier-peak token prices ($30/MM input, $180/MM output) used in the blended $80/MM-token inference revenue calculation. Replace with the project team's finalized per-release price curve when it lands. benchlm.ai/llm-pricing

External references used to anchor TBD/sensitivity inputs in this report:

## Tolling daily MWh cap (TOLL_DAILY_CAP_PEAKER / INTERMEDIATE / NEAR_NAMEPLATE)

- EIA Today in Energy (June 2023) - U.S. simple-cycle natural gas turbines operated at record highs in summer 2022. Anchors the peaker bracket: SCGT fleet annual capacity factor 9.6-14.1% (2017-2023), ~17% summer / ~10% off-season, ERCOT among regions above the national average. eia.gov/todayinenergy/detail.php?id=55680

- EIA Form 923 (plant-level operations data) - net generation and heat input by prime-mover, used to verify the SCGT capacity-factor range above and to derive an MDQ-equivalent envelope. eia.gov/electricity/data/eia923

- Modo Energy - ERCOT BESS tolling agreement explainer. Establishes that current ERCOT tolling contracts give the offtaker direct dispatch rights over the full asset envelope (near-nameplate convention). modoenergy.com/.../ercot-explainer-part-one

- GridStor - Fortune 500 tolling pact for a 150 MW / 350 MWh ERCOT battery (Dec 2025). Recent comparable disclosing contract structure for the near-nameplate bracket. stocktitan.net/.../gridstor-tolling-agreement

- Entergy MUCPA - Capacity Sale and Tolling Agreement RFP (template). Reference document showing standard daily-MDQ contract structure for natural-gas-fired tolling. rfp.entergy.com/.../MUCPA-Tolling.pdf

## Forward-curve drift (--gas-drift-pct / --power-drift-pct)

- EIA Short-Term Energy Outlook (May 2026) - natural gas section. Baseline: 2025 actual HH = $3.53/MMBtu, 2026 forecast = $3.50/MMBtu (essentially flat - hence default gas_drift_pct = 0). 2026 LNG export forecast = 17.0 Bcf/d (up from 15.1 in 2025), near peak export-complex capacity. eia.gov/outlooks/steo/report/natgas.php

- EIA STEO May 2026 (full PDF) - the primary source for the Brent forecast that drives the geopolitical-shock overlay. Reports Brent $106/b May-Jun 2026 and $115/b 2Q peak amid Strait of Hormuz disruption; ~10.5 Mbbl/d of Mideast production shut-in. eia.gov/outlooks/steo/pdf/steo_full.pdf

- EIA Today in Energy - Henry Hub spot prices to fall slightly in 2026, rise in 2027. Confirms the EIA's view that higher 2026 Brent supports associated-gas production (bearish HH), partially offsetting the LNG-pull (bullish HH) channel. eia.gov/todayinenergy/detail.php?id=67004

- EIA Press Release (May 12, 2026) - STEO revision amid continued Mideast disruption. Quantifies the Hormuz-closure assumption (closed until late May, shipping resumes June) used to size the +30% Brent shock scenario. eia.gov/pressroom/releases/press588.php

- Rigzone (May 20, 2026) - EIA lowers Henry Hub forecast for 2026, 2027. Documents the May-vs-April STEO revision ($3.67 -> $3.50/MMBtu) - useful for justifying baseline gas_drift_pct = 0 against a contemporaneous-news critique. rigzone.com/.../usa-eia-lowers-henry-hub-price-forecast

- S&P Global - Texas summer 2025 power prices may top 2024 on weather and strong gas. Anchors the HH -> ERCOT LMP elasticity: Houston Ship Channel summer forwards ~$4/MMBtu (vs ~$2 in 2024) coincided with ERCOT Houston on-peak forwards $110-167/MWh (vs DAM $30-40/MWh in 2024). spglobal.com/.../texas-summer-power-prices-2025

- Potomac Economics - 2024 State of the Market Report for ERCOT. Source for implied heat-rate ranges (Figure 7) used to derive the HH->LMP pass-through elasticity (~0.5 in gas-on-margin hours). potomaceconomics.com/.../2024-State-of-the-Market-Report.pdf

## AI-structural drift baseline (--gas-drift-pct 0.005 / --power-drift-pct 0.01)

Separate driver from the Brent/geopolitical scenarios above: secular load growth from data-center buildout. Translated from the sources below into a roughly +1 %/yr LMP, +0.5 %/yr HH structural drift, prorated to ~half a year for the June-December 2026 horizon.

- ERCOT Capacity, Demand and Reserves (CDR) Report - December 2024 release. Forecasts peak demand growth ~5-7 %/yr through 2030 driven heavily by data-center and crypto load, with reserve margins tightening. Primary anchor for the 'AI-structural' load-growth scenario. ercot.com/.../CDR_December2024.pdf

- EIA Annual Energy Outlook 2026 - Reference case for ERCOT shows electricity sales growth ~3-4 %/yr and a continued role for gas-fired generation on the margin. Translates to roughly +2 %/yr on average LMP and +1 %/yr on HH in the project horizon. eia.gov/outlooks/aeo

## Token-price decay halflife (TOKEN_PRICE_HALFLIFE_DAYS = 270 days)

- benchlm.ai - LLM Pricing Trends. Headline finding: the curated frontier Price Index fell from 100 at the GPT-4 launch (March 2023) to 5.5 in April 2026, a 94.5 percent decline over ~37 months. Equivalent to log2(100/5.5) ~= 4.18 halvings, so halflife ~= 37/4.18 ~= 8.85 months ~= 270 days. Primary anchor for TOKEN_PRICE_HALFLIFE_DAYS. The page also reports the largest single step-down: GPT-4 output $60 -> $10 via GPT-4o replacement (~5.4-month halflife, the dramatic-jump extreme). https://benchlm.ai/llm-pricing-trends

- data/benchlm-pricing-with-dates.csv - per-tier corroboration computed from the joined vendored snapshot. Log-linear fits give: OpenAI flagship (GPT-4 Turbo -> GPT-4o -> GPT-4.1 -> GPT-5) ~12-month halflife; Anthropic Opus (Claude 3 -> 4.1 -> 4.5) ~19-month halflife with one large step-down; Anthropic Sonnet (3.5 -> 4 -> 4.5) flat over 16 months. The 270-day default sits closest to the OpenAI flagship trajectory; the Sonnet observation explains why the sensitivity sweep extends to 540 days.

- data/revenue_model_inputs/revenue_model_assumptions.csv - team-supplied halflife recommendation (60 days, LogNormal median 60, p10 35, p90 120). This is DERIVED under uplift_factor = 1.5: their CSV note states 'Frontier launch-price transitions imply roughly 53 day median and 73 day mean under uplift_factor=1.5.' Under our smaller METR-anchored cadence-dependent uplift (1.105 at 30d, 1.219 at 60d), the same frontier-price-transition data implies a substantially longer halflife - in the same neighborhood as our 270d default from the benchlm Price Index. The two anchors are reconcilable under a single uplift-halflife decomposition.

## Per-release quality multiplier (cadence-dependent: uplift_factor = 2 ** (cadence_days / 210))

- METR - 'Measuring AI Ability to Complete Long Tasks' (March 2025). Headline finding: the length of tasks that frontier-model agents can autonomously complete with 50 percent reliability has roughly doubled every ~7 months over the past 6 years, through November 2025 (Claude 3.7 Sonnet at ~1-hour horizon). Translating to the project's 60-day release cadence: 2^(60/210) ~= 1.22x per release. Primary anchor for UPLIFT_FACTOR_DEFAULT. https://metr.org/blog/2025-03-19-measuring-ai-ability-to-complete-long-tasks/

- a16z - 'Generative AI Enterprise 2024' (16-company survey of enterprise AI spending). Reports 2-5x year-over-year growth in enterprise AI spend through 2024 ($1.5-2B 2023 run-rate -> $5B+ end of 2024). Used as a sanity-check bracket on the uplift factor: at the low end (2x annual spend growth) per-release uplift = 2^(60/365) ~= 1.12x; at the high end (5x) per-release uplift = 5^(60/365) ~= 1.30x. The METR-anchored 1.22x falls within this range. https://a16z.com/generative-ai-enterprise-2024/

- Anthropic Opus 4.1 release notes (August 2025) - 'Pricing is the same as Opus 4'. Confirms the operative observation that list price per token has been FLAT at the flagship Opus tier across multiple releases, validating the doc_blended frame (price-flat-to-falling x quality-uplift = net per-release multiplier). https://www.anthropic.com/news/claude-opus-4-1

- data/revenue_model_inputs/revenue_model_assumptions.csv - team-supplied uplift recommendation 1.25 (LogNormal median 1.25, p10 1.08, p90 1.65, sensitivity 1.10 / 1.25 / 1.50 / 1.80). Basis quoted from the CSV: 'Benchmark deltas imply smaller general-document uplift than 1.5x; 1.5x remains aggressive case for coding/reasoning.' Our cadence-dependent values (1.105 at 30d, 1.219 at 60d, 1.346 at 90d) all sit inside their recommended 1.10-1.80 bracket and bracket their 1.25 median.

# Report

(Sections below to be written; planning sections above describe the model setup and serve as input.)

## Executive Summary

## Model Setup

### Inference + Training

Data Inputs

Assumptions

Constraints

### Power

Data Inputs

Assumptions

Constraints

### Workflow (Descriptive Model) & The Code

## Results / Sensitivities

## Analysis

## Conclusion