# Project description

Optimization of a 6-month operating policy (1 June 2026 → 1 December 2026) for a pair of 100-MW data centers in the ERCOT region — Houston (HB_HOUSTON) and West Texas (HB_WEST) — covering allocation of compute capacity between training and inference, timing of new model releases, and procurement of grid power. The framework combines a deterministic linear program (LP) solving each hour's decisions for a given (training cadence, procurement) policy with a Monte-Carlo wrapper that simulates joint price paths and selects the policy maximizing expected profit under uncertainty.

# Project setup

## 3 main decisions

- Training vs. inference compute split each hour at each site

- Power procurement at each site: RT LMP vs. tolling (Houston only — no gas plant available at West) and BESS dispatch (charge from grid, discharge to data center, or sell back to grid)

- Frequency of frontier-model retraining (cadence in days). Higher cadence keeps the deployed model closer to peak revenue but takes compute away from inference; lower cadence preserves inference capacity but lets revenue decay

# Assigned Tasks and Deliverables

The prescribed task entails employing real-options analysis to design an economically viable operating policy for allocation and timing for incurring training cost as well as power procurement during the next 6 months (starting 1 June 2026 to 1 December 2026) with corresponding profitability (i.e., volatility-dependent conditional payoff) estimates during this period.

- Detailed description of the process of analysis, motivation/rationale justifying the methodology, and all assumptions.

- Value and estimate breakeven as well as marginal profitability increases for tolling fixed-lease-rate battery storage (BESS) at each location with recommendations as to under what market conditions either or both of these tolling arrangements would be economically viable.

- Substantive description of wind-solar intermittency in the context of MIHR citing the ERCOT data portal, and the contingency of the prescribed unscheduled outage scenarios.

# Training + Inference

## Training

### Power and computational requirements

Each model release k has a compute requirement that we project from its training-start date in three steps: training_start_date → projected parameter count → projected training FLOPS → required compute-MWh.

### Parameter projection

Parameter counts in notable AI systems have grown exponentially. We fit a log-linear regression of log10(params) on Excel-serial date using the Epoch AI "Parameters in notable artificial intelligence systems" dataset with a post-2020 filter. The post-2020 cut produces a steeper, more recent trend that better matches frontier model sizes; the post-2010 alternative is documented in the code (`fit_growth_curves.py`) for comparison.

Post-2020 fit (n = 395 observations, R² ≈ 0.30):

log10(params) = −49.6585 + 1.32043 × 10⁻³ · day_serial

That gives a parameter-count doubling time of ~228 days. We then apply a 5× competitiveness multiplier (planning-doc convention) so projections track 2026 frontier-class models (DeepSeek 1.6T, GPT-class 1.7T, etc.) rather than the dataset-wide average. The CSV is dated 2023, so the 5× bridges 2023 → 2026 frontier.

Projected post-multiplier parameter counts at planning-doc reference dates:

| Release | Training start | Params (post-5×) | Pre-multiplier |
| --- | --- | --- | --- |
| R1 (initial) | 2026-06-01 | 1.02 × 10¹² | 2.04 × 10¹¹ |
| R2 | 2026-06-22 | 1.09 × 10¹² | 2.18 × 10¹¹ |
| R3 | 2026-08-22 | 1.31 × 10¹² | 2.62 × 10¹¹ |
| R4 | 2026-10-22 | 1.58 × 10¹² | 3.16 × 10¹¹ |
| R5 | 2026-12-22 | 1.90 × 10¹² | 3.80 × 10¹¹ |

These figures are derived in `assumptions.project_params(training_start_date)`.

### FLOPS projection

Training compute follows a power law in parameter count. We fit log10(petaFLOPS) vs. log10(params) on the Epoch AI "Training computation vs. parameters" dataset (n = 457, R² ≈ 0.84):

log10(petaFLOPS) = −9.8192 + 1.7187 · log10(params)

A 1.72 exponent matches Chinchilla-style scaling (FLOPS ∝ params × tokens, with tokens ∝ params). With the post-2020 parameter projection and the 5× multiplier, the projected training-FLOPS per release is:

| Release | Training start | Training petaFLOPS | FLOPS |
| --- | --- | --- | --- |
| R1 | 2026-06-01 | 6.64 × 10¹⁰ | 6.64 × 10²⁵ |
| R2 | 2026-06-22 | 7.41 × 10¹⁰ | 7.41 × 10²⁵ |
| R3 | 2026-08-22 | 1.02 × 10¹¹ | 1.02 × 10²⁶ |
| R4 | 2026-10-22 | 1.40 × 10¹¹ | 1.40 × 10²⁶ |
| R5 | 2026-12-22 | 1.93 × 10¹¹ | 1.93 × 10²⁶ |

Derived in `assumptions.project_petaflops(training_start_date)`.

### Hardware and FLOPS → MWh conversion

Each data center holds 90,000 H-100-class GPUs at 80 MW of compute capacity (180,000 GPUs and 160 MW compute across both sites). H-100 SXM5 and H-100 PCIe have different sustained throughputs; SXM is generally better for training. We model an aggregate effective rate built from a configurable SXM/PCIe split:

| Parameter | Default value | Notes |
| --- | --- | --- |
| H100_SXM_SUSTAINED_TFLOPS_PER_SEC | 500 TF/s | ≈ 50 % of FP8-dense theoretical 989 TF/s. ⚠ TBD |
| H100_PCIE_SUSTAINED_TFLOPS_PER_SEC | 380 TF/s | ≈ 50 % of FP8-dense theoretical 756 TF/s. ⚠ TBD |
| SXM_FRACTION_DEFAULT | 0.60 | 60 % SXM / 40 % PCIe. ⚠ TBD |
| FLOPS_PER_COMPUTE_MWH | 1.83 × 10²¹ | Derived: GPUs × (split · TF/s) × 3600 / 80 MW |

Aggregate sustained throughput at 100 % compute utilization across both sites = 81.4 PFLOP/s = 2.93 × 10²³ FLOPS/hour. Dividing the per-release FLOPS by FLOPS_PER_COMPUTE_MWH gives the compute-MWh needed for that release. Multiplying by PUE = 1.25 converts to grid-MWh, which is what the LP's `train` and `inf` variables track.

### Three core decisions, recap

- Training cadence (days between releases). Treated as a discrete sweep over candidate values, evaluated in expectation over MC price paths.

- SXM vs PCIe split. Currently collapsed into a single aggregate FLOPS_PER_COMPUTE_MWH because the underlying per-workload sustained rates are flagged TBD in the planning doc. Easy to split when finalized.

- Hourly training/inference split per site. The LP picks these endogenously given the cadence and price paths.

### First release: 100 % compute

The initial release R1 begins 1 June 2026 and runs at 100 % compute utilization on both sites — no inference revenue accrues during R1's window. R1's window length is therefore set by compute-need / max-compute-rate:

R1_duration = project_compute_mwh(2026-06-01) / (2 × 80 MW · 24 h) ≈ 36,289 / 3,840 ≈ 9.5 → 10 days

All subsequent releases R(k+1) start at R(k)'s release date and run for the chosen cadence_days, with both training and inference allowed in those windows.

## Inference

### Basic constraints

- Inference can only use compute capacity not currently being used for training. Implemented as the LP's per-site compute cap: train + inf ≤ 100 grid-MWh/hr (= 80 compute-MWh/hr).

- Token revenue at any hour must cover marginal power cost. The LP enforces this implicitly via its profit objective: an additional MWh of inference is only taken if its revenue exceeds its grid cost.

- Maximum request rate is such that power/computational cost does not exceed that unused by training, in any hour.

### Revenue per MWh

Per the RFP, a 100 % allocation of 80 MW compute to inference produces 5 trillion tokens/day per site. This pins our conversion factor:

TOKENS_PER_COMPUTE_MWH = 5 × 10¹² / (80 × 24) ≈ 2.604 × 10⁹ tokens/cMWh

Token pricing follows GPT-5.4Pro on benchlm.ai with the RFP-specified 2/3 input + 1/3 output revenue mix:

blended rate = (2/3) × $30 + (1/3) × $180 = $80 per million tokens

That implies an inference revenue per compute-MWh of $208,333 (at peak, no decay); after PUE conversion, $166,667 per grid-MWh. This is the peak revenue an hour earns when the deployed model is at its newest. Per-hour revenue then declines via the token-decay path described below.

### Token-price decay between releases

From the planning-doc observation that frontier token prices halve every couple of months, we model the per-hour inference revenue rate as:

rev_inf[h] = base_rev × multiplier_of_current_release × 0.5 ^ (days_since_release / 60)

Where `current_release` is the most-recent release as of hour h. Halflife defaults to 60 days; sensitivity to 30/45/60/90/120 days is provided in `halflife_sensitivity.py`.

### Per-release multipliers (token cost / quality)

Each release carries a `token_revenue_multiplier`. Four schemes are available, configurable per-run:

| Scheme | Multiplier rule | Captures |
| --- | --- | --- |
| constant | 1.0 for every release | No generation-over-generation differentiation; only the decay sawtooth matters |
| quality_uplift | uplift_factor^k for release k | Each new release captures more value because it's a better model |
| market_decay | 0.5^(release_t_days/60) | Each new release earns less because the consumer market price has fallen |
| doc_blended | quality_uplift × market_decay | Planning-doc story: quality rises but market deflates simultaneously |

Default for the headline run is `doc_blended` with `uplift_factor = 1.5`.

### Revenue at the hour level

Total inference revenue per hour at a site, given the LP picked inf grid-MWh of inference:

revenue[h, s] = inf[h, s] × rev_inf[h]

Aggregated across hours and sites and netted against power costs (LMP/toll/BESS-charge) and BESS lease (if applicable), this is the LP's objective.

# Conversion (compute → grid-MWh)

The LP's training and inference decision variables are in grid-MWh — the same units as the power-procurement variables — because compute capacity converts to grid power at PUE = 1.25. Key relationships:

| Quantity | Formula | Value (default) |
| --- | --- | --- |
| Power Use Effectiveness | Grid MWh / Compute MWh | 1.25 |
| Per-site grid power cap | RFP | 100 MWh/hr |
| Per-site compute capacity | Grid cap ÷ PUE | 80 compute-MWh/hr |
| Both-site grid cap | 200 MWh/hr | = 4,800 MWh/day |
| Both-site compute cap | 160 compute-MWh/hr | = 3,840 cMWh/day |
| FLOPS per compute-MWh | Σ_GPU type frac · TF/s · 3600 / 80 MW | 1.83 × 10²¹ |
| Tokens per compute-MWh | 5T tokens/day ÷ (80 MW · 24 h) | 2.60 × 10⁹ |

Constraints (planning doc):

- Per-site grid draw g_lmp + g_toll ≤ 100 MWh/hr × capacity_factor

- Aggregate grid draw across both sites ≤ 200 MWh/hr

- Per-site train + inf ≤ 100 grid-MWh/hr (≡ 80 compute-MWh/hr after PUE)

- Aggregate train + inf ≤ 200 grid-MWh/hr (≡ 160 compute-MWh)

- Compute-clearing: g_lmp + g_toll + dis_dc = train + inf (LP balance equation)

- RFP daily training floor: Σ train per day, summed across both sites, ≥ 500 MWh-grid

# Power

## Grid LMP (HB_HOUSTON, HB_WEST)

Hourly Day-Ahead Market settlement-point prices for the two ERCOT hubs are sourced from the ERCOT ISO publishing site (xlsx in `data/`). For the 6-month horizon, we use the actual 2025 June-November DAM hourly prices, shifted forward one calendar year, as a deterministic proxy. For the headline real-options analysis these prices are replaced by N Monte-Carlo paths simulated from a seasonal Ornstein-Uhlenbeck model calibrated on the full 2025 actuals.

Note: the RFP requests RT-LMP. DAM is the closest available proxy in the repo; RT is typically more volatile and would slightly increase the value of tolling and BESS arbitrage.

## Henry Hub gas (for tolling)

Daily Henry Hub spot prices from the EIA are used in the Houston tolling cost formula. Loaded from `HH_full.csv`.

## Tolling at Houston (period-long option, hourly exercise)

Tolling is structured as a binary period-long contract decision (`Scenario.use_houston_tolling`). When the contract is signed, the LP gets an hourly option to draw up to TOLL_MAX_MW of power at the tolling cost — exercised only when LMP > toll cost. Toll variable cost per MWh:

toll_cost[$/MWh] = (HH_gas[$/MMBtu] + $3/MMBtu O&M) × 9.5 MMBtu/MWh ≈ $57/MWh at $3 gas

Heat rate is 9,500 BTU/kWh per the RFP — typical of an efficient simple-cycle peaker. West Texas has no tolling option (no gas plant available).

Tolling parameters (currently default-modelled with no fixed surcharge):

| Parameter | Default | Notes |
| --- | --- | --- |
| TOLL_HEAT_RATE_BTU_PER_KWH | 9,500 | RFP — simple-cycle peaker |
| TOLL_VOM_PER_MMBTU | $3/MMBtu | RFP — variable O&M premium |
| TOLL_MAX_MW | 100 MWh/hr | Hourly cap = site grid cap (assumption) |
| TOLL_MAX_MWH_PER_DAY | None | ⚠ TBD daily cap (RFP refers to one; value not specified) |
| TOLL_FIXED_SURCHARGE_PER_MWH | $0 | ⚠ TBD capacity payment (RFP didn't specify) |

## BESS (40 MW / 160 MWh / 92 % RTE at each site)

Battery storage is modeled as a tolling agreement (physical or virtual) for 6 months. The contract is binary per site (`Scenario.bess_sites` can be any subset of {HOUSTON, WEST}). When active at a site, the LP gets four hourly decisions:

- ch[h, s]: MWh charged from the grid (drawn at LMP)

- dis_dc[h, s]: MWh discharged to the data center (substitutes for grid draw at LMP)

- dis_grid[h, s]: MWh discharged and sold back to the grid at LMP

- soc[h, s]: state of charge at hour h, bounded 0 ≤ soc ≤ 160 MWh

BESS dynamics:

soc[h+1] = soc[h] + √0.92 × ch[h] − (dis_dc[h] + dis_grid[h]) / √0.92

Charge and combined discharge each capped at 40 MWh/hr; round-trip efficiency 92 % (= per-direction √0.92 ≈ 0.959). The 6-month lease per site is amortized from the RFP-stated cost structure:

lease/site = $60M capex / 15 yr / 2 + $2M opex/yr / 2 = $3M for 6 months

Or $6M total if BESS is active at both sites. This fixed cost is excluded from the LP objective directly (it's a constant) but subtracted in `cost_breakdown` so the per-scenario profit numbers reflect it.

# Model Dynamic

## Decision variables

Per hour h ∈ {0..4391} and per site s ∈ {HOUSTON, WEST}:

| Variable | Units | Bounds | Meaning |
| --- | --- | --- | --- |
| g_lmp[h, s] | grid-MWh | 0 .. 100 | Power drawn at RT LMP |
| g_toll[h, s] | grid-MWh | 0 .. 100, 0 at WEST | Power drawn under Houston tolling |
| train[h, s] | grid-MWh | ≥ 0 | Compute allocated to training |
| inf[h, s] | grid-MWh | ≥ 0 | Compute allocated to inference |
| ch[h, s] | MWh | 0 .. 40 (BESS sites) | BESS charge from grid |
| dis_dc[h, s] | MWh | ≥ 0 | BESS discharge to data center |
| dis_grid[h, s] | MWh | ≥ 0 | BESS discharge sold at LMP |
| soc[h, s] | MWh | 0 .. 160 | BESS state of charge |

## Model-variable parameters

- Training cadence (days between releases). Default candidate set: [10, 15, 20, 25, 30, 45, 60, 75, 90, 120, 150, 180]. Filter retains those whose smallest non-initial release's natural training rate is ≥ 500 MWh-grid/day AND whose largest release's required rate is ≤ 4,800 MWh-grid/day. Typical filtered set: [25, 30, 45, 60, 75, 90].

- Token-multiplier scheme (per-release revenue scaling). Default `doc_blended`.

- Procurement scenario (use_houston_tolling, bess_sites). Default everything-on; Phase C optimizes.

- Number of Monte-Carlo paths. Default 50 for production runs.

## Objective

Maximize expected profit across MC price paths:

max  Σ_h Σ_s ( rev_inf[h] · inf[h, s]  +  LMP[h, s] · dis_grid[h, s] )

− Σ_h Σ_s ( LMP[h, s] · g_lmp[h, s]  +  toll_cost[h] · g_toll[h, s]  +  LMP[h, s] · ch[h, s] )

− BESS_6mo_lease × |bess_sites|

## Constraints

- Per-site grid draw cap: g_lmp + g_toll ≤ 100 MWh/hr × capacity_factor

- Per-site compute cap: train + inf ≤ 100 MWh-grid/hr (= 80 compute-MWh/hr)

- Power balance: g_lmp + g_toll + dis_dc = train + inf (LP balance; ensures power into the DC matches compute load)

- Tolling at Houston only: g_toll[h, WEST] = 0

- Houston toll capacity: g_toll[h, HOUSTON] ≤ TOLL_MAX_MW (× capacity_factor)

- Optional daily toll MWh cap: Σ_h g_toll[h, HOUSTON] per day ≤ TOLL_MAX_MWH_PER_DAY (TBD)

- BESS power: ch ≤ 40 MWh/hr, dis_dc + dis_grid ≤ 40 MWh/hr

- BESS SOC: 0 ≤ soc ≤ 160 MWh, soc[h+1] = soc[h] + √0.92 × ch − (dis_dc + dis_grid) / √0.92

- BESS starts and ends at SOC = 0 (no inventory at horizon endpoints)

- Per-release training: Σ_h Σ_s train[h, s] over R_k's window ≥ project_compute_mwh(R_k.start) × PUE

- R1 initial: inf[h, s] = 0 for all h in R1's window (100 % compute → training)

- Outside any release window: train[h, s] = 0 (unless schedule is empty, in which case the daily floor governs)

- RFP daily training floor (mandatory): Σ_s,h-in-day train[h, s] ≥ 500 MWh-grid/day

## Optimization workflow (run_planning_doc.py)

Profit is maximized in four sequential phases:

### Phase A — Cadence selection (procurement = all on)

For each candidate cadence c (after filter), solve the LP across N MC price paths in parallel via a ProcessPool. Mean per-path profit is the score for c. Stage 1 sweeps the filtered broad set; Stage 2 refines six cadences ±30 % around the Stage-1 winner. Winner is the cadence with highest mean profit.

### Phase B — Locked-cadence diagnostic (procurement = all on)

At the Phase-A winning cadence, re-solve the LP on every path and report averaged-across-paths metrics: cost breakdown, profit distribution, procurement mix. This is a marginal-cost view — the LP "uses" BESS even when the lease is net-negative because the lease is a sunk constant from the LP's perspective.

### Phase C — Procurement optimization (full-cost view, lease included)

At the locked cadence, sweep eight procurement combinations: toll ∈ {off, on} × BESS-sites ∈ {none, Houston, West, both}. For each combo, solve N paths and average profit including the BESS lease as a fixed-cost subtraction. Pick the combo with highest mean profit; this is the optimal procurement under full-cost accounting.

### Verification — Cadence under optimal procurement

Re-solve a tight ±30 % cadence neighborhood under Phase C's optimal procurement. Confirms the cadence winner doesn't shift when negative-NPV options are stripped out (in practice never does, because cadence-vs-cadence profit gaps are ~$3 B while procurement-vs-procurement gaps are ~$5 M).

### Final output

Re-solve the LP on every MC path with the (verified cadence, optimal procurement) policy and save:

- Per-path hourly schedule in long format (all decision variables: g_lmp, g_toll, train, inf, compute-MWh, FLOPS, tokens, BESS dispatch if enabled).

- Averaged-across-paths hourly schedule with ±std for every variable.

- Per-cadence mean/std/percentile summary CSV.

- Console: first 24 hours at HOUSTON with mean ± std annotations.

## Inputs at a glance

| Group | Item | Source / Default |
| --- | --- | --- |
| Power | Hourly RT LMP (HB_HOUSTON, HB_WEST) | ERCOT DAM 2025, shifted +1 yr; MC paths from OU calibration |
| Power | Tolling cost | (HH_daily + $3/MMBtu) × 9.5 MMBtu/MWh |
| Power | BESS 6-month lease per site | $3 M (= $60 M / 15 yr / 2 + $2 M / 2) |
| Power | Facility cap | 200 MWh/hr (= 100/site) |
| Compute | Compute cap | 160 cMWh/hr (= 80/site after PUE 1.25) |
| Compute | Hardware throughput | FLOPS_PER_COMPUTE_MWH = 1.83 × 10²¹ |
| Compute | Min daily training | 500 MWh-grid/day, system-total (mandatory) |
| Training | Param projection | log10(P) = −49.66 + 1.32e-3·day (post-2020 fit, 5× multiplier) |
| Training | FLOPS projection | log10(petaFLOPS) = −9.82 + 1.72·log10(P) |
| Training | R1 fixed | Starts 6/1/2026, 100 % compute, ~10 days |
| Revenue | Token price (peak) | $30/MM input + $180/MM output ⇒ $80/MM blended |
| Revenue | Tokens per compute-MWh | 2.604 × 10⁹ |
| Revenue | Decay halflife | 60 days (sensitivity available) |

# Report

(Sections below to be written; planning sections above describe the model setup and serve as input.)

## Executive Summary

## Model Setup

### Inference + Training

Data Inputs

Assumptions

Constraints

### Power

Data Inputs

Assumptions

Constraints

### Workflow (Descriptive Model) & The Code

## Results / Sensitivities

## Analysis

## Conclusion